- Department of Electrical Engineering

- EE-5040: Adaptive Signal Processing

April 2010			Computer Simulation Assignment #2			10marks

Consider the adaptive channel equalisation model as shown in the figure below (see also pg. 224 in the E-copy of Prof.Ali Sayed’s “Adaptive Filter Theory” for a similar problem). Here, the independent, uniformly distributed data symbols I(k) entering the channel F(z) are drawn from a 16-QAM alphabet, and the transmit signal power E[|I(k)|2] = σ12 =1. The 3-tap channel is specified by F(z)= 1- 0.80z-1+ 0.64z-2 and the additive Gaussian noise component v(k) is zero mean with variance σv2= 0.02. The linear equaliser has order M=14, and the desired response d(k)=I(k-), where the decoding delay =5.

d(k)=I(k-)

F(z)





I(k)

v(k)

u(k)

y(k)

e(k)

{wm}

m=0,1..M-1

I(k)

(a) Determine the LMSE (Wiener) solution wopt. What is the corresponding Jmin?

(b) Simulate the Least-Mean Squares (LMS) algorithm based adaptive equaliser with μ=0.10 being the gain constant. Plot the error convergence curve by averaging e2(k) over 25 Monte-Carlo runs, where the average squared error is defined by ξ(k)= (1/25)  . Plot 10log10(ξ(k)) versus k, for k=1,2….2000. What is the simulated and theoretical excess MSE (EMSE) that you get?

(c) For the same channel conditions and M and , find the “best possible” gain constant μ for the LMS that will converge within 1500 samples. Plot its convergence curve. What are the EMSE values here?

(d) Repeat part (b), using now a Recursive Least Squares (RLS) algorithm to define the adaptive equaliser. Use forgetting factor =0.995 and initial choice of inverse of the data covariance P(0)=100. What is the simulated and theoretical excess MSE (EMSE) in this case?

(e) For the same =0.995, if you are allowed to change P(0), specify a new P(0) that will make RLS converge at least five times faster than the selection in (d). What are the EMSE values here?

(f) Make a single consolidated plot, with both the LMS curves in “solid blue”, and both of the RLS curves in “solid black”. Mark also the Jmin line on this graph.

(g) To this consolidated plot, you are to add one more convergence curve corresponding to the algorithm that you choose from the list provided in the next page. The corresponding 10log10(ξ(k)) versus k is to be marked in “solid red” colour on the same plot.

(h) Specify salient points about the parameter values(s) you considered for this “chosen” algorithm in part (g). What is the simulated EMSE for the same?

Choice of algorithms for part (g) of the assignment

1. -Normalised LMS 	(see pp. 183-184 in E-copy for 1. thro 8.)

2. -NLMS with power normalisation

3. Sign-error LMS

4. Leaky LMS

5. Least Mean Fourth (LMF)

6. Least Mean Mixed Norm (LMMN)

7. Affine Projection (APA and -APA) – pg. 192

8. Partial Rank Algorithm (PRA and APA) – pg.197

Variable Step-Size (VSS) Algorithms (see scanned page on URL from A.Sayed’s book pg. 255)

9. VSS – LMS

10. RVS – LMS

11. KVS – LMS

12. VSS – NLMS

13. Gauss-Newton algorithm (pg. 256-257 in E-copy) – similar to RLS

Blind Algorithms

14. Sato Algorithm (see Haykin 4th Edition)

Constant Modulus Algorithm & Godard Algorithm

15. CMA(1,2) and NCMA – see pp.188 in E-copy for 15. and 16.

16. CMA(2,2)

17. CMA(1,1) – see Haykin 4th Ed for 17. and 18.

18. CMA(2,1)

19. Reduced Constellation Algorithm (RCA) – pp.187

20. Multi Modulus Algorithm (MMA) – pp. 188

21. Stop & Go Algo. (Picchi & Pratti, T-Comm, 1987 – see reference in E-Copy p-768)

22. Benvenisti & Goursat Algorithm (T-Comm, 1984 – see reference in E-Copy p-759)

23. Any other adaptive algorithm of your choice (if your own, provide proper justification!)

Note: Sign-up sheet is on the office door at ESB-334B. Please sign up a first choice and second choice before 6.00pm today (Monday, April 26).