Department of Electrical Engineering

EE5040 – Adaptive Signal Processing

March 15, 2010			Mid-term Quiz (Open Book)				          25marks

1. [6 marks] Consider the measurement model below, where a linear MSE (Wiener) estimator is to be defined for the equalizer W(z) which will minimize E[e2(n)].  Here, input i(n) is white noise with unit variance, and additive noise v(n) has variance σv2= 0.10 and is uncorrelated with i(n).

e(n)

y(n)

0.36/(1-0.8z-1)

W(z) =w0+w1z-1

u(n)

+

1+2z-1

i(n)

d(n)

v(n)

+

-

(a) Find the correlation matrix R and the cross-correlation vector p.

(b) What is the LMSE estimate for W(z)?

(c) What is the Jmin for this LMSE problem?

2. [6 marks] Consider a Mth order FIR filter w whose input vector u(n) and output y(n) are related by y(n)=wHu(n). Let z(n)=R-1/2u(n) where R is the MxM auto-correlation matrix corresponding to u(n). Let z(n) be input to another Mth order FIR filter v so as to obtain x(n)=vHz(n).

(a) Specify v so that the two outputs, y(n) and x(n), are identical.

(b) Describe a steepest-descent algorithm (SDA) for updating v (i.e., v(n)).

(c) What are the time constants associated with the learning curve (convergence curve) of this SDA?

(d) How does this compare with the Newton’s algorithm?

3. [6 marks] In a certain model, the measurements are given by u(n)=x(n)+v(n) where the signal component x(n)=0.9x(n-1)-0.5x(n-2)+q(n), with the driving noise q(n) being white with variance 1. The additive noise component v(n) is also white with variance 0.20, and is mutually uncorrelated with q(n).

(a) Set up a 1-step predictor of order M using the LMS algorithm. Hint: Use desired response d(n)=u(n).

(b) If a step size =0.01 is used, what will be the excess MSE of the LMS algorithm for M=5? (Use the small step-size approximation if required).

(c) What will be the excess MSE if a -NLMS is used instead, with ’=0.1? State your assumptions in deriving this result.

(d) For what choice of  will the LMS have the same excess MSE?

4. [7 marks] Consider the following 7 measurements {u[k]} and the desired signal {d[k]}. Also assume that u[0]=2.

| k | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| u[k] | -2 | -2 | 2 | -2 | 2 | 2 | 2 |
| d[k] | -2.5 | -1.5 | 2.3 | -2.4 | 2.9 | 1.3 | 1.1 |

- Find the least squares estimate wLS = [w0 w1]T using the Covariance formulation.

- Starting with P(0)=10I and wRLS (0)=[0 0]T find wRLS (2), P(2), and e(2)? Use =1.

KG/IITM/Mar.2010