EE-5060 Communication Techniques

Nov. 3, 2010					       Quiz #2 				                         30 Marks

1. [4 marks] Trunk node #1 with M=8 servers accepts E=4Erlangs of traffic, while trunk node #2 with the same number of servers, accepts only 3Erlangs of traffic. Which of the two nodes has a better output utilization factor  = (1- Pb)E / M, where Pb is the blocking probability of the given node?

2. [8 marks] Given a population of N=10,000 users, each offering Eu=0.02 Erlangs of traffic, define a 3-stage blocking switch with k sub-arrays in the middle-stage, each containing 200x200 cross-points such that the blocking probability Pb≤10−4.

(a)  Use the Lee graph approach to find this least value of k.

(b) Determine the number of cross-points for the above switch.

(c) For the same value of k as found in (a), it is decided to resize the sub-arrays in the three stages so as to minimize the number of cross-points. Can you provide at least one other choice for n (or m) that will have a lower complexity, but again ensuring thatPb≤10−4? What is this new complexity and the new Pb?

3. [5 marks] A signal of bandwidth 4 MHz is radiated through a 15dBi antenna with a power of 250milliWatts. The carrier frequency is 1 GHz, and the receive antenna has a gain of 6dBi.

(a) For a required SNR at the detector input of 6dB, and given the receiver noise figure of 5dB, find the receiver sensitivity in dBm. (Assume that the thermal noise power density is -174dBm/Hz.)

(b) Assuming a path-loss exponent n=4, what is the maximum shadow loss (in dB) that can be tolerated by the receiver at a distance of d=200m from the transmitter?

4. [5 marks] The received signal in a given application is to be amplified by a cascade of two amplifiers, where the gains and noise-figures are as follows: A1 with F1= 0.32A1; and A2=20dB with F2=51. Notice that for the first amplifier in the chain, increasing the gain will also increase it’s noise figure. Determine the choice of A1 that will provide the least overall noise figure for this cascade. What is this overall F in dB?

5. [8 marks] A cellular decides to employ 12-cell reuse, which is basically a 4-cell reuse pattern, followed by using 3 sectored antennas over each 1200 sector in the hexagonal cell of side R=1km. The path loss exponent is n=3, and the antenna gain pattern (in dB versus angle) is as shown in the figure below:

-1800

1800

-1200

11200

-600

600

2dB

20dB

6dB

Use suitable assumptions to answer the following:

(a) Plot the 12-cell reuse pattern over a sufficient number of cells. Hint: Use 4-cell coloring first and then do the sectoring to get the 12 colors.

(b) Find the best case signal to interference ratio (SIR) in dB.

(c) What is the worst-case SIR in dB? Justify your answers to (b) and (c).

K.Giridhar, IITM, Nov.2010